# III.  Documentation

During a cast, a CTD Cast log sheet needs to be filled out in order to document all aspects of the cast.  When the cast is finished, this log sheet is placed in the log book for scientists to refer to for cross-referencing their water samples.  The log sheet shown here is an example from an LTER cruise; each cruise may want to develop their own log sheet.

The log sheet can contain as much information as desired and can be used in a variety of ways, although there is some critical information that needs to be filled out:

| CTD (cast number) | The number of the cast on that particular cruise. It also serves to identify the data file associated with that cast. |
| --- | --- |
| Event Number or station number | Science event number. Call the bridge to get this cast’s event number. (This may be specific only to certain cruises.) |
| Grid | Where the CTD was cast. This can also differ per cruise. |
| Latitude/Longitude | Used to verify where on the grid the CTD was cast. This is important if the cast is located between set stations or if the computer has trouble getting the information from the AshTech GPS. |
| Depth to bottom | Helps characterize the cast. |
| Technician | Who was running the cast from the Dry Lab. |
| Data File Name | Imperative in order to find the data from that particular cast. |
| Sensors | In case a sensor had to be replaced or added before this cast. If no sensors have been replaced, simply check “No” in all the boxes. |
| Target Bottle Depths | This grid helps the technician identify which bottles to fire at which depth, and to help keep the rosette balanced. |
| Underway Log | Most important are time and pressure, as well as listing which bottle was fired at that particular depth, the temperature at the surface, and the mixed layer depth. |

Always place the finished log sheet in the log book.  An example of a log sheet (2 pages) are on the following pages.

LTER Ice Cruise			NBP 99-06		CTD: 0013

Event: 00030					Station:		Grid:240.100

|  | Date y/m/d | GMT | Latitude | Longitude | depth (uncorr) | depth (corrected) |
| --- | --- | --- | --- | --- | --- | --- |
| Begin | 99/06/18 | 13:44 | 6652'26" | 7050'59" | 521 |  |
| Bottom | 99/06/18 | 13:57 | 6652'32" | 7051'03" | 520 |  |
| End | 99/06/18 | 14:11 | 6652'32" | 7051'06" | 521 |  |

| Technician: | IANNUZZI |
| --- | --- |
| File Name: | LTER013 |
| Audio Tape: | CTD013 |
| Disk: |  |

Sensors

|  |  | New? |  |
| --- | --- | --- | --- |
|  | Serial Number | Yes | No |
| Pressure 1 |  |  | X |
| Pressure 2 |  |  | X |
| Temperature 1 |  |  | X |
| Temperature 2 |  |  | X |
| Conductivity 1 |  |  | X |
| Conductivity 2 |  |  | X |
| Oxygen 1 |  |  | X |
| Oxygen 2 |  |  | -- |
| Pump 1 |  |  | X |
| Pump 2 |  |  | X |
| Transmissometer |  |  | X |
| Scatterometer |  |  | X |
| Flourometer |  |  | X |
| Altimeter |  |  | X |

Comments

In pancake ice.

BCT=Yes E Word=0001

Bring unit in to check

Fixed—Restart cast

BCT=Yes AGAIN Right at surface—CHECK!!

CTD Log Sheet side 1

LTER Ice Cruise			NBP 99-06		CTD:

Target  Bottle Depths

| 1 BOT | 2 XX | 3 XX | 4 | 5 XX | 6 XX | 7 | 8 XX | 9 XX | 10 | 11 XX | 12 XX |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 13 PYC | 14 XX | 15 XX | 16 PYC | 17 XX | 18 XX | 19 ML | 20 XX | 21 XX | 22 SFC | 23 XX | 24 XX |

Underway Log

| Time | Pressure | Temp | Salinity | Bottle # | Bottle Trips & Comments |
| --- | --- | --- | --- | --- | --- |
| : |  |  |  |  | Surface Enter Water |
| : |  |  |  |  | Surface Start Down |
| 13:57 | 479 |  |  |  | 1, Surface Temp |
| 14:01 | 335 |  |  |  | 4, Mixed layer depth |
| 14:03 | 241 |  |  |  | 7 |
| 14:05 | 167 |  |  |  | 10 |
| 14:07 | 104 |  |  |  | 13 |
| 14:08 | 67 |  |  |  | 16 |
| 14:09 | 48 |  |  |  | 19 |
| 14:11 | 9 |  |  |  | 22 |
| : |  |  |  |  |  |
| : |  |  |  |  |  |
| : |  |  |  |  |  |
| : |  |  |  |  |  |
| : |  |  |  |  |  |
| : |  |  |  |  |  |
| : |  |  |  |  |  |
| : |  |  |  |  |  |
| : |  |  |  |  |  |
| : |  |  |  |  |  |
| : |  |  |  |  |  |
| : |  |  |  |  |  |
| : |  |  |  |  |  |
| : |  |  |  |  |  |
| : |  |  |  |  |  |
| : |  |  |  |  |  |
| : |  |  |  |  |  |
| : |  |  |  |  |  |
| : |  |  |  |  |  |
| : |  |  |  |  |  |
| : |  |  |  |  |  |
| : |  |  |  |  |  |

CTD Log Sheet side 2